Chemical Hazard Classification Taxonomy

23-Category GHS-Aligned Labeling Schema for Industrial Safety AI Models

| Author: | Busari Quadri Oyelekan |
| --- | --- |
| Document: | DOC-001 / Rev. 3.2 |
| Date: | 2024 |
| Status: | Portfolio Work Sample — Confidential |

# 1. Overview and Purpose

This taxonomy was developed to support the annotation of chemical safety datasets for machine-learning-based hazard recognition systems. The schema draws directly on my background in industrial chemistry and practical experience with GHS (Globally Harmonised System of Classification and Labelling of Chemicals) as applied in laboratory and manufacturing environments.

The 23-category framework was built iteratively across 12 specialized annotation projects, starting with a narrower 14-category version that proved insufficient for distinguishing sub-classes of reactive and environmental hazards. Each revision incorporated edge cases flagged during QA review.

# 2. Taxonomy Structure

Categories are organised into five top-level divisions. Each division contains between two and seven sub-categories reflecting distinct hazard mechanisms, labeling conventions, or regulatory significance.

## Division A - Physical Hazards (7 categories)

| ID | Category | Definition | GHS Pictogram / Signal Word |
| --- | --- | --- | --- |
| A1 | Flammable Liquids | Liquids with flash point below 60°C. Includes gasoline, ethanol, acetone. | Flame / DANGER (FP < 23°C) or WARNING |
| A2 | Flammable Solids | Solids that combust readily or contribute to fire by friction. | Flame / WARNING |
| A3 | Self-Reactive Substances | Thermally unstable substances liable to undergo exothermic decomposition. | Exploding bomb or Flame / DANGER |
| A4 | Pyrophoric Substances | Liquids or solids that ignite spontaneously in air within 5 minutes. | Flame / DANGER |
| A5 | Oxidising Agents | Substances that cause or contribute to the combustion of other materials. | Flame over circle / DANGER or WARNING |
| A6 | Compressed Gases | Gases under pressure in containers, including liquefied and dissolved gases. | Gas cylinder / WARNING |
| A7 | Explosives | Substances capable of rapid gas release causing damage through pressure. | Exploding bomb / DANGER |

## Division B: Health Hazards (8 categories)

| ID | Category | Definition | Key Indicators |
| --- | --- | --- | --- |
| B1 | Acute Toxicity (Oral) | Fatal or harmful if swallowed. Classified LD50-based into categories 1-4. | LD50 value, skull pictogram, DANGER/WARNING |
| B2 | Acute Toxicity (Dermal) | Fatal or harmful in contact with skin. Assessed via dermal LD50. | Skull or exclamation pictogram |
| B3 | Acute Toxicity (Inhalation) | Fatal or harmful if inhaled. LC50-based assessment. | LC50 value, route of exposure noted |
| B4 | Skin Corrosion/Irritation | Irreversible or reversible skin damage on application. | Corrosion or exclamation pictogram |
| B5 | Serious Eye Damage/Irritation | Irreversible damage to eye tissue or reversible corneal inflammation. | Corrosion or exclamation pictogram |
| B6 | Respiratory Sensitisers | Cause hypersensitivity of airways following inhalation. | Health hazard pictogram / DANGER |
| B7 | Carcinogenicity | Known or suspected to induce cancer. IARC Group 1/2A/2B. | Health hazard / WARNING or DANGER |
| B8 | Reproductive Toxicity | Adverse effects on sexual function, fertility or foetal development. | Health hazard / DANGER or WARNING |

## Division C: Environmental Hazards (3 categories)

### C1 - Acute Aquatic Toxicity

LC50 or EC50 value ≤ 1 mg/L for fish, crustaceans, or algae. Applies to substances that cause immediate harm to aquatic organisms. Most commonly encountered with industrial solvents, heavy metal compounds, and chlorinated hydrocarbons.

### C2 - Chronic Aquatic Toxicity

Substances not classified as acutely toxic but capable of causing long-term effects at ≤ 1 mg/L. Includes substances with low degradability or bioaccumulation potential.

### C3 - Ozone-Depleting Substances

Halogenated compounds that catalyse stratospheric ozone destruction. Classified per Montreal Protocol Annexes. Rare in modern industrial use but encountered in legacy equipment.

## Division D - Mixed/Combination Hazards (3 categories)

These categories cover substances presenting multiple simultaneous hazard classifications. Annotators must assign the primary hazard first and note secondary classifications in the metadata.

### D1 - Flammable + Toxic

Substances presenting both flash point below 60°C and LD50-based toxicity. Common examples include methanol (toxic, flammable) and many organophosphate solvents.

### D2 - Corrosive + Oxidising

Compounds with dual corrosive and oxidising properties. Concentrated sulphuric acid and nitric acid are typical representatives.

### D3 - Sensitiser + Carcinogen

Substances that are both respiratory/skin sensitisers and classified carcinogens. Requires dual GHS pictogram labeling and separate annotation entries.

## Division E - Special Regulatory Categories (2 categories)

### E1 - Substance of Very High Concern (SVHC)

Substances meeting REACH Annex XIV criteria. Includes CMR substances (carcinogenic, mutagenic, reprotoxic), PBT (persistent, bioaccumulative, toxic), or vPvB substances. Requires specific regulatory labeling distinct from standard GHS.

### E2 - Unlabelled/Unknown Hazard

Applied to substances lacking complete SDS documentation or where hazard classification is pending review. This category was added at revision 2.1 following repeated edge cases during the first industrial equipment project.

# 3. Annotation Decision Rules

The following rules govern category selection in ambiguous cases:

- When a substance meets criteria for both Division A and Division B, annotate both and set primary category to whichever the SDS lists first under Section 2.

- Division D categories take priority over individual A or B categories when the combination hazard is explicitly noted in the SDS.

- E2 (Unknown Hazard) should only be applied when three or more mandatory SDS sections are missing or redacted.

- Molecular structure annotations accompany every entry in categories B4 through B8 - structure image must be IUPAC-compliant.

- GHS pictograms must be annotated using the standardised SVG library provided in the project toolkit, not freehand.

- Signal word (DANGER vs WARNING) must be captured as a separate metadata field from the pictogram category.

# 4. Inter-Annotator Agreement Notes

During QA auditing across 12 projects using this taxonomy, the following categories consistently produced the lowest IAA scores and required the most guideline clarification:

- B6 vs B7 (Respiratory Sensitiser vs Carcinogen) - resolved by adding the rule that B6 requires confirmed bronchial hypersensitivity evidence in the SDS, not merely respiratory route of exposure.

- C1 vs C2 (Acute vs Chronic Aquatic Toxicity) - resolved by making degradability the deciding factor when both acute and chronic data are present.

- D-category assignment - resolved by adding mandatory dual-entry requirement so substances are recorded in both their primary single-hazard category and the appropriate Division D combination category.

Following these clarifications, IAA for this taxonomy reached 97.2% across a review batch of 4,400 annotations.

# 5. Revision History

| Version | Date | Changes | Author |
| --- | --- | --- | --- |
| 1.0 | Jan 2023 | Initial 14-category schema - Divisions A and B only. | BQ Oyelekan |
| 2.0 | Apr 2023 | Added Division C (Environmental Hazards). Expanded B to 8 categories. | BQ Oyelekan |
| 2.1 | Jul 2023 | Added E2 (Unknown Hazard) following edge case batch review. | BQ Oyelekan |
| 3.0 | Nov 2023 | Added Division D (Combination Hazards). Dual-entry rule introduced. | BQ Oyelekan |
| 3.2 | Mar 2024 | IAA threshold raised to 97%. Clarified B6/B7 and C1/C2 decision rules. | BQ Oyelekan |